| CHANGE REQUEST |  |
| --- | --- |
| Request identity number: | C00264 |
| Type of request: | New |
| Request status level: | For evaluation |
| Proposal: | Symbol S00472 is to be replaced by a new symbol as attached and the references of application notes A00254 and A00266 to be updated. Note that also the name is proposed to be changed. The name of S00472 is not logical; although the switch is intended to work on one of the two supply circuits to e.g. a lamp (on the “two pole side” between two switches of the type S00471 (two way single pole switch)), the device as such is bipolar and not single pole. |
| Reason: | This change request is part of MT 60617 recommendation to harmonize the graphical symbol family "switches for installation diagrams" to make all titles, graphical presentations, descriptions, application notes to be consistent and help users of symbols to choose right symbols particular purpose. During the preparation of this proposal MT 60617 was consulted. SE NC propose to replace S00472 with the new symbol shown below |
| Requested by: | Per-Åke Svensson pasv@pasv.se SE |

| SYMBOL |  |  |  |
| --- | --- | --- | --- |
| Symbol identity number: | S01915 | Status: | Proposed |

| Drawing: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Earlier published in: | . | (IEC 60617-11 (ed.2.0) 11-14-07) |  |  |
| Name: | EN FR | Intermediate pole changer, two poles Permutateur intermédiaire, deux pôles |  |  |
| Keywords: | EN | installations in buildings, switches |  |  |
| Applies: Application note: |  | S00466, S00024 A00266, A00254 |  |  |
| Application class: |  | Installation diagrams |  |  |
| Function class: |  | Q Controlled switching or varying, S Converting a manual operation into a signal |  |  |
| Shape class: |  | Circles, Lines |  |  |
| APPLICATION NOTE |  |  |  |  |
| Note identity number: | A00254 | Status: | Standard |  |
| Note type: | General |  |  |  |

Note (EN):

The graphical symbol S01915 represents a pole changing device (switch) in installation diagrams which has a single actuator. The function of the device is illustrated in figure 1.

Figure 1. Circuit diagram presentation on symbol S00472

Applies to: S00472